# Lab 7 Report - ADC Sampling, Aliasing, and False Telemetry

## Name:

- Lab Partners:

Complete the blanks using measurements; include units. Content 70 + Evidence 15 + Conclusion 15 = 100 points. The optional RC extension is not required.

Board / station / sketch version: ____________________    Measured Vref: __________ V

Scope / generator IDs: ____________________    Probe factor: ______    Scope sample rate: __________

D8 rising-edge interval: mode 2 __________ ; mode 5 __________ . Actual rate = 1/interval: mode 2 __________ ; mode 5 __________ .

Check each sine capture: overruns = 0, near_rail_warning = NO, valid frequency. If not, record the warning, correct the setup, and capture again. Ignore the tone/alarm status during DC calibration.

# Table 1 - ADC Calibration (15)

| DC input | DMM input (V) | Predicted code | Mean code | Code error | Observation |
| --- | --- | --- | --- | --- | --- |
| Low: about 1 V |  |  |  |  |  |
| High: about 3 V |  |  |  |  |  |

Show step size and both code predictions using measured Vref. Define code error as reported mean minus predicted code.

# Table 2 - Sampling and Aliasing (25)

Use sine, 1.00 Vpp, +2.50 V offset, High Z display; verify about 2-3 V on the scope. Capacitor disconnected. Commands: 2 = lower rate, 5 = higher rate, c = capture again.

| Condition / set input | Sample rate (samples/s) | Nyquist limit (Hz) | Predicted frequency (Hz) | Nano frequency (Hz) | Scope frequency (Hz) |
| --- | --- | --- | --- | --- | --- |
| Faithful: 200 Hz; send 2 |  |  |  |  |  |
| Alias: 1,800 Hz; send 2 |  |  |  |  |  |
| Higher rate: 1,800 Hz; send 5 |  |  |  |  |  |

Show the alias prediction and absolute frequency error for each row: abs(Nano frequency - scope frequency). Use fresh valid captures.

# Table 3 - Toy Alarm and Control (30)

Use sine, 1.00 Vpp, +2.50 V offset, High Z display; verify about 2-3 V on the scope. Capacitor disconnected. Commands: 2 = lower rate, 5 = higher rate, c = capture again.

| Condition / command | Scope frequency (Hz) | Nano frequency (Hz) | Nano Vpp (mV) | Alarm state | Frequency error (Hz) |
| --- | --- | --- | --- | --- | --- |
| Baseline: 400 Hz; 2 |  |  |  |  |  |
| Substituted: 1,800 Hz; 2 |  |  |  |  |  |
| Higher rate: 1,800 Hz; 5 |  |  |  |  |  |
| Positive check: 200 Hz; 5 |  |  |  |  |  |
| Restored: 400 Hz; 2 |  |  |  |  |  |

Compare measured error before/after the higher-rate control. Explain the positive check and the restored baseline. Record ON, OFF, or INVALID exactly as reported.

Alarm ON requires valid apparent frequency 180-220 Hz and at least 500 mVpp. INVALID is not evidence of successful suppression. Describe any warnings: ____________________

Optional RC notes (ungraded): at each frequency compare source-side Vpp with A0-side Vpp; report reduction and whether the genuine 200 Hz positive check remained valid.

# Evidence (15)

Insert readable, labeled photos or screenshots for these three evidence groups. Expand this section as needed; retain settings and warnings.

## DC Calibration

Higher-point DMM voltage, Nano mean code, and reference voltage.

## Analog Input versus Alias

Scope frequency and the matching lower-rate Nano capture, including rate, warnings, and apparent frequency.

## Toy Alarm and Control

A labeled comparison of all five Table 3 conditions, including the genuine 200 Hz positive check. Multiple readable images are fine.

# Conclusion (15)

Summarize what you did and learned. Then answer these questions using your measurements:

- Why could the scope and Nano report different frequencies for the same physical input? Use your measured values and alias prediction.

- How did the higher sampling rate change frequency error and the toy alarm? What did the positive check establish?

- Why is this a single-source model rather than proof of an attack on a real sensor? What access would a real attacker need?

- State one remaining limitation: for example, could a genuine unwanted 200 Hz input still satisfy the rule? Why does a valid ADC reading not establish trusted origin?