# Lab 4 Report - Recover the Hidden Message

## Name:

- Lab Partners:

# Table 1 - Components, Settings, and Prediction (15)

Record the values actually used. Use resistance mode only while the circuit is disconnected from every source.

| Quantity or setting | Nominal | Tolerance | Measured / used | Unit | Calculation / note |
| --- | --- | --- | --- | --- | --- |
| Resistor R | 10 | ±0.5 |  | Ω |  |
| Inductor L | 2.9 | ±0.05 |  | mH |  |
| Inductor winding resistance | 5 | approximately |  | Ω |  |
| Capacitor C | 1.00 | ±0.02 |  | µF |  |
| Generator amplitude | 2 | fixed |  | Vpp |  |
| Generator offset | 0 | fixed |  | V |  |

Show the resonance calculation:  f0 = 1 / (2 pi sqrt(LC)) = ____________________

Predicted resonant frequency: ____________________   Expected component-tolerance range: ____________________

# Table 2 - Normalized RLC Band-Pass Response (35)

Use your team's predicted f0 from Table 1. For each row, calculate target frequency = multiplier x f0, set that target on the generator, and record the measured frequency from the oscilloscope.

| f / f0 | Target f | Measured f | Vin (Vpp) | Vout (Vpp) | H = Vout / Vin |
| --- | --- | --- | --- | --- | --- |
| 0.05 |  |  |  |  |  |
| 0.20 |  |  |  |  |  |
| 0.50 |  |  |  |  |  |
| 0.75 |  |  |  |  |  |
| 1.00 |  |  |  |  |  |
| 1.25 |  |  |  |  |  |
| 2.00 |  |  |  |  |  |
| 5.00 |  |  |  |  |  |
| 10.0 |  |  |  |  |  |
| 20.0 |  |  |  |  |  |

Measured peak frequency: ____________________   Peak H = Vout / Vin: ____________________

Graph your measured H = Vout / Vin values against f / f0. Use an XY scatter plot with a logarithmic horizontal axis from 0.05 to 20 and a linear vertical axis. Paste or draw the completed graph below.

How does your graph show that this circuit is a band-pass response? Why might the measured peak differ from 1.00 f0?

# Table 3 - Hidden-Message Recovery (20)

| Condition | What you heard or decoded | Direct observation / measurement | Interpretation |
| --- | --- | --- | --- |
| Unfiltered direct connection |  |  |  |
| Filtered RLC output |  |  |  |
| Decoded repeated word |  |  |  |

What changed after inserting the filter, and what evidence makes the decoded word more defensible than a guess?

# Evidence (15)

Insert a readable photo or screenshot for each scenario:

## Series-RLC Circuit

Pictured: C, L, R, source, common, and both probe locations before enabling the sweep.

## Peak Response

Pictured: Input and resistor output at the measured peak with frequency, amplitudes, scales, and probe factors readable.

## Message Recovery

Pictured: Composite input and filtered output with both channels, scales, amplitudes, and station connections readable.

# Conclusion (15)

Give a summary of what you did in this lab. What was easy to do? What was difficult? What did you learn this lab? In addition, answer these questions:

- What was the measured RLC pass frequency, and how closely did it agree with the prediction?

- How did Vout / Vin change below, near, and above the measured pass frequency?

- Why did the Morse-code word become easier to recognize after filtering? Cite one direct observation or measurement.

- What interference would this filter not remove, and what does this result not establish about the interference source or intent?