# Lab 3 Report - Transformers & Breadboard Crosstalk

## Name:

Lab Partners:

# Table 1 - Intentional Transformer Coupling (25)

Use resistance mode only while the transformer is disconnected. Use AC-voltage mode for powered measurements; do not use a current jack.

| Measurement | DMM mode | Measured value | Expected / comparison |
| --- | --- | --- | --- |
| Primary winding resistance |  |  |  |
| Secondary winding resistance |  |  |  |
| Primary-to-secondary resistance |  |  |  |
| Primary AC voltage Vp |  |  |  |
| Secondary AC voltage Vs |  |  |  |
| Voltage ratio Vs / Vp |  |  |  |

What do the cross-winding resistance result and the measured voltage ratio each demonstrate?

________________________________________________________________________________

________________________________________________________________________________

# Table 2 - Crosstalk Baseline and Orientation (20)

Keep source, resistors, probe factors, scope scales, bandwidth, and acquisition settings fixed while changing conductor orientation.

| Condition | Source Vpp | Neighbor Vpp | Vneighbor / Vsource | What changed? |
| --- | --- | --- | --- | --- |
| Source disabled |  |  |  |  |
| Parallel baseline |  |  |  |  |
| 90-degree crossing |  |  |  |  |
| Restored parallel baseline |  |  |  |  |

Did the 90-degree crossing increase or reduce crosstalk? Cite the measurements and explain why restoring the parallel baseline matters.

________________________________________________________________________________

________________________________________________________________________________

# Table 3 - Neighbor-Line Resistance and Crosstalk (25)

Restore the parallel baseline. Change only the neighbor-line resistor and keep the source, conductors, and scope settings fixed.

| Neighbor condition | Resistance | Source Vpp | Neighbor Vpp | Reduction vs. high R (%) |
| --- | --- | --- | --- | --- |
| High-value neighbor resistor |  |  |  |  |
| Low-value neighbor resistor |  |  |  |  |

How did lower neighbor-line resistance affect measured crosstalk? State the quantitative evidence and one design tradeoff.

________________________________________________________________________________

________________________________________________________________________________

# Evidence (15)

Insert readable photographs or screenshots for the required evidence points:

## Transformer Measurement

Pictured: Transformer setup, DMM AC-voltage reading, and station card.

## Parallel-Wire Baseline

Pictured: Parallel source and neighbor conductors with readable Channel 1 and Channel 2 measurements.

## Lowest-Crosstalk Condition

Pictured: The tested orientation or neighbor-resistance condition that produced the lowest repeatable neighbor-line voltage.

# Conclusion (15)

Summarize what you did and learned, then address each prompt:

How did the transformer demonstrate intentional coupling without a direct DC path between windings?

How did you verify that the unwanted waveform on the neighbor line came from the source line?

Which tested change reduced crosstalk most, and what quantitative evidence supports that conclusion?

Name one controlled variable and explain why it was necessary for a fair comparison.

State one broader claim about interception, isolation, or signal containment that this evidence does not prove.

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________