# Lab 8 Report — Break the Toy Door

## Name:

- Lab Partners:

Preliminary report | Content 70 + Evidence 15 + Conclusion 15 = 100 points. Record actual board output. CRC calculation uses the supplied helper; no polynomial division is required.

Board / device ID: __________   Firmware: __________   Date: __________

Evidence source: [ ] live Arduino   [ ] instructor-provided fallback. Browser check matches are not target acceptance or flags.

# Table 1 — Port Discovery (10 points)

Record the board's values. Explain why decoding hex is not decrypting a secret.

| Item | Observed value | Evidence / notes |
| --- | --- | --- |
| Board / firmware / team ID |  |  |
| Baud / serial format / line ending |  |  |
| Hex clue and decoded service address |  |  |
| Bytes 0–7: field meanings |  |  |
| Stage-1 parity convention |  |  |

# Table 2 — Stage 1 — Parity (15 points)

Restore the captured baseline before each error test. Identify which bit(s) changed. Record the stage-1 flag and transition.

| Trial | Sent frame (8 bytes) | Expected check | Arduino response | LED / state |
| --- | --- | --- | --- | --- |
| Captured LOCK |  |  |  |  |
| One extra-byte bit changed |  |  |  |  |
| Two extra-byte bits changed |  |  |  |  |
| OPEN + matching parity |  |  |  |  |

Stage 1 flag from board: __________________________________

Parity: [ ] even [ ] odd. Bit(s) changed in extra byte: one-bit test __________ ; two-bit test __________ . Why did the two tests differ?

# Table 3 — Stage 2 — CRC (20 points)

Distinguish changing the attached CRC from finding a fixed-CRC collision. Record the stage-2 flag and transition.

| Trial | Sent frame (8 bytes) | Calculated CRC | Arduino response | LED / state |
| --- | --- | --- | --- | --- |
| Captured LOCK |  |  |  |  |
| OPEN + unchanged old CRC |  |  |  |  |
| OPEN + recomputed CRC |  |  |  |  |

Stage 2 flag from board: __________________________________

CRC covers bytes ______ through ______. Why could the modified OPEN frame pass after recomputation?

# Table 4 — Stage 3 — Approved CRC Collision (25 points)

Record target CRC, locked digits, adjustable bytes, candidates tested, and stage-3 flag. Show that the final payload differs but its CRC is unchanged.

| Trial | Sent frame (8 bytes) | Calculated CRC | Arduino response | LED / state |
| --- | --- | --- | --- | --- |
| Approved LOCK capture |  |  |  |  |
| OPEN + valid but unapproved CRC |  |  |  |  |
| Different OPEN + approved CRC |  |  |  |  |

Stage 3 flag from board: __________________________________

Approved target CRC: ______   Adjustable bytes: ______   Candidates tested: ______

Locked digit positions / fields: ________________________________________

What changed between LOCK and OPEN, and what stayed fixed?

# Evidence (15 points)

Provide three readable evidence groups, with captions showing device ID, stage, sent packet, actual response, and LED state. Add pages if needed. Do not invent flags for a fallback run.

## Parity and port discovery

Startup identity plus one parity rejection and a successful stage-1 command, with decoded service-sheet address.

## CRC upgrade

Changed command rejected with its old CRC and accepted after CRC recomputation.

## Whitelist collision

This team's approved baseline, fixed-CRC search constraints/result, and board response proving the OPEN action.

# Conclusion (15 points)

Answer each prompt in 2–3 sentences (5 points each). Connect your claim to your recorded evidence.

What is different about parity, recomputing a CRC, and preserving a CRC through a collision? Use one of your packets in the explanation.

Why did the CRC whitelist fail to establish permission? Separate a helper check match from the board's command acceptance.

What should the controller check to establish message authenticity, and what additional check addresses replay? State one limitation of this toy USB/LED model.